MMT OBSERVATORY 6.5 m TELESCOPE

Doc.No. : Issue : a Date : May 11, 2007

MMT Observatory

6.5m  TELESCOPE

MMT Rotator Analysis

Maestro load case

|  | Signature | Date |
| --- | --- | --- |
| Prepared |  |  |
| Reviewed |  |  |
| Approved |  |  |

- Revision History

| Issue | Date | Changes | Responsible |
| --- | --- | --- | --- |
| a | May 11, 2007 | Start analysis | Shawn Callahan |

- Table Of Contents

1.	Revision History	3

2.	Table Of Contents	4

3.	About this document	5

3.1.	Purpose	5

3.2.	Reference Documents	5

4.	FEA model results	6

4.1.	Loads	6

4.2.	Boundary conditions	6

4.3.	Model generation.	7

4.4.	Equivalent stress- Von Mises	8

4.5.	Strength of materials	11

4.6.	Stripping strength of flange	11

4.7.	Conclusion	11

- About this document

  - Purpose

This document contains the analysis of the stress in the MMT rotator bearing due to an overhung load from Maestro’s counterweight.

  - Reference Documents

See the MMT SiteScape for all documents.

- FEA model results

The following results show the Von-Mises stress in the races and rollers of the MMT instrument rotator. All loads have been applied to a plate bolted to 5 of the ¾-16 tapped holes in the instrument mounting surface of the inner race.

  - Loads

According to David Dean on 5/8/07, the counterweight weighs 850 lbs [390 kg.]. Measurements of the cart indicate that the load ranges from 2 to 3 feet from the bolt circle on the rotator. To be conservative I applied 850 lbs 33 inches from the bolt circle using a 1-inch steel plate.

  - Boundary conditions

Fixed supports were applied to the outer race surface that mounts to the telescope. Since the central cone provides approximately 30 inches of section this constraint should be a good approximation of the stiffening from the telescope cell.

  - Model generation.

Since the highest stress lies in the rollers, the mesh was refined for the threes rollers showing the highest stress.

  - Equivalent stress- Von Mises

The maximum stress is in the rollers. With the above load case the maximum stress is around 700 psi.

I also looked at two additional load cases where the load was applied over a 12-inch wide plate .

The stress from 850 lbs applied to a 12-inch wide plate is still below 2000 psi.

Finally, I looked at the “monkey wrench” load of the entire counterweight supported on a 1” wide “wrench” applying 850 lbs over a 33-inch moment arm. This is obviously a worst case load to the rotator.

The maximum stress was still less then 13,000 psi under the “wrench”. The bearing stress was less than 2300 psi.

  - Strength of materials

The rollers are AISI 52100 Steel heat treated to 58-62 RC. This high strength material has a yield strength of 295,000 psi.

The material of the race is AISI 4150 steel heat treated to 250 BHN (minimum). The yield strength of this material is around 120,000 psi.

  - Stripping strength of flange

The flange has 24 ¾-16 UNF tapped holes 1.5 inches deep. Using a conservative stripping strength of 90 ksi, each bolt should support 40,000 lbs.

  - Conclusion

If we applied all of the weight of Maestro’s counterweight to a single bolt on the rotator flange (NO, I DO NOT RECOMMEND THIS!) we would not damage the inner and outer race, bearing rollers, or tapped holes.

--oOo--

Doc_info_start

Title: Fill in title here

Document Type: Report

Source: Steward Observatory

Issued by: Joar Brynnel

Date_of_Issue: Fill in date here

Revised by:

Date_of_Revision:

Checked by:

Date_of_Check:

Accepted by:

Date_of_Acceptance:

Released by:

Date_of_Release:

File Type: MS Word

Local Name: Repeat title here

Category: Fill in category here (3 digits)

Sub-Category: Fill in sub category here (3 digits)

Assembly: Fill in Assembly name here

Sub-Assembly:

Part Name:

CAN Designation: Fill in full doc number here

Revision: Fill in revision letter here

Doc_info_end